RF2EGA.COM    UPDATE

EQUIPMENT:

When using "RF2EGA.COM" your computer must be fitted with an

EGA card (or compatible card), which is attached to either a

high resolution  monochrome monitor, or a EGA color monitor.

Either monitor must be capable of  a resolution of 640 X 350

lines.

SOFTWARE DIFFERENCES:

RF2EGA uses  350 screen lines to display the fax pictures on

as opposed  to 200  screens lines that the CGA version uses.

The result  of this  is that  higher resolution fax pictures

are displayed on your video monitor.

When you  are in  a RECEIVE  WINDOW  a certain number of fax

lines will  be superimposed  on each screen line in order to

squeeze the  total number  of fax  lines that you are saving

onto the  350 line  screen. When  you are  saving a total of

1400 fax  lines or less, 4 fax lines will be superimposed on

each screen  line. When  you are  saving more  than 1400 fax

lines  in   each  buffer,  then  5  lines  of  fax  will  be

superimposed on  each screen  line. This tends to vertically

squeeze the picture in the RECEIVE WINDOW (also the RE-ALIGN

WINDOW) when you have a buffer size greater than 1400 lines.